Template only MUST modify to site conditions

| Site / Area: |  | Date of Assessment |  | Risk Assessment # | 038RA |
| --- | --- | --- | --- | --- | --- |
| Completed by (name) |  | Signature |  |  |  |
| In Consultation with: |  | Signature |  |  |  |
| Identify / describe activity, equipment, area or event you are assessing: |  |  | AIR COMPRESSOR |  |  |
| Authorised by: |  | Signature: |  | Date: |  |
| In conjunction with this risk assessment, training / education and development of a relevant SOP may be required. |  |  |  |  |  |
| #### Step 1: Identify the hazard/s / Impact: #### What do you believe are the hazards? What could happen? |  | Step 2: Assess the potential risks: What do you believe are the risks? How could this happen? |  | Step 3: Reducing the risk: What do you believe can be done to reduce the risk? Controls |  |
| Machinery & Equipment - Parts being ejected - Cracked hoses - Uncontrolled release of compressed air - Being hit by a moving part - Lack of maintenance |  | - Becoming entangled in moving parts of the plant (“V” belt drive wheels) - Laceration or amputation of fingers or hand in the moving belt/wheels - Being hit by damaged air-line / fitting becoming dislodged - Blowing compressed air at others or self - Injured while conducting maintenance on the plant - Plant could explode / disintegrate from over-pressure valve or regulator not working correctly. |  | - Plant has mesh guarding fitted to drive wheels and is enclosed in a mesh safety cage. - Regular workplace inspections are conducted - Person instructed to not blow compressed air at others and not to place air outlet on their skin. - Persons are trained in plant maintenance, and all equipment is “locked-out” electrically when maintenance is undertaken - Blow off fittings to be used where possible. - Lock Out / Tag Out system in place. - Two (2) yearly inspections required for those compressors that require registration with SafeWork SA. (Refer WHS Regulations SA Schedule 5 (Part 1)) |  |
| Electrical - electrical exposure - frayed cords - faulty plugs - overheating equipment - cutting through cords - faulty appliance. |  | - Electrocuted - Electric shock from poorly maintained / damaged plant - Electrical installation could be impacted and damaged - Damaged wiring could cause an electrical fire - Plant could start automatically without warning. |  | - The plant undergoes regular electrical inspections (test & tag) - The main power distribution board is RCD protected - Workplace inspections are conducted to identify defective items - Plant is fitted with standard on / off control panel. - Lock Out / Tag Out system in place. |  |
| Noise - loud noises - prolonged exposure - noisy equipment. |  | - Potential hearing loss /impairment - Workers and others cannot communicate due to noise |  | #### PPE is available if required, ear plugs, earmuffs. - Compressor has been permanently mounted outside the building. |  |
| Hazardous Manual Tasks - moving small air compressors |  | - Sprains / strains |  | #### Minimise the need to move portable air compressors. - Wheels are checked regularly to ensure are working effectively |  |
| Gravity - slips, trips or falls from cords |  | - Sprains / strains - Broken limbs |  | - Ensure cords are kept secured and not in walkways. |  |
| Other |  |  |  |  |  |
| Review hazard / risk assessment if task or circumstances change & at intervals appropriate to the level of risk (minimum 5 years) |  |  |  |  |  |

| Step 4: Monitor & Review: |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Were the controls effective? |  |  |  | Were there any unforeseen hazards / incidents? |  |  |  | New controls |
| Yes |  | No |  | Yes |  | No |  |  |