| Site / Area: |  |  |  | Date of Assessment |  |  |  | Risk Assessment # |  | 011RA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Completed by (name) |  |  |  | Signature |  |  |  |  |  |  |
| In Consultation with: |  |  |  | Signature |  |  |  |  |  |  |
| Identify / describe activity, equipment, area or event you are assessing: |  |  |  |  |  | CLOTHES DRYER |  |  |  |  |
| Authorised by: |  |  |  | Signature: |  |  |  | Date: |  |  |
| In conjunction with this risk assessment, training / education and development of a relevant SOP may be required. |  |  |  |  |  |  |  |  |  |  |
| #### Step 1: Identify the hazard/s / Impact: #### What do you believe are the hazards? What could happen? |  |  |  | Template only MUST modify to site conditions Step 2: Assess the potential risks: What do you believe are the risks? How could this happen? |  |  |  | Step 3: Reducing the risk: What do you believe can be done to reduce the risk? Controls |  |  |
| Gravity - Incorrect installation |  |  |  | - Dryer could dislodge from wall / support structure causing serious injury |  |  |  | - Dryer installed by competent person, used and maintained in accordance with manufacturer’s instructions and safety guards and warning signs are in place. |  |  |
| Electricity - Faulty Equipment |  |  |  | - Disruption to power supply - Electric shock - Burns - Electrocution |  |  |  | - Dryer is regularly tested and tagged (if not hard wired). - RCD installed at main switchboard and checked regularly – push button and operating time tests. - Regular servicing of dryer by a competent person. Repairs & modifications by competent person only - Dryer is isolated from the power supply prior to maintenance work. - Electrical Certificate of Compliance (COC) provided where electrical work undertaken. - If a Commercial Dryer – hard wired through an isolating switch. |  |  |
| Hazardous Manual Tasks - Repetitive or sustained force (carrying/pushing/pulling items) - High or sudden movement (a struggling patient) - Repetitive movement - Sustained or awkward posture - Vibration |  |  |  | - Musculoskeletal injuries - Sprains - Strains - Repetitive Strain Injury (RSI) - Slips, trips, falls |  |  |  | - Dust, lint, water is cleaned up regularly. - Mop and bucket available to clean spills. - Dryer located at a reasonable height to make it easier to put clothes in and remove them. - Trolleys and baskets are available to place laundry into and move. - Ensure workers have attended manual handling training. |  |  |
| Airborne Contaminants - Dusts |  |  |  | - Respiratory illness i.e. asthma, allergies etc. |  |  |  | - Dust, lint is cleaned up regularly. - Lint filters are cleaned before each use. - Dryer is on preventative maintenance plan to ensure vents are cleaned at least annually. - Conduct inspections of hoses and vents to check for blockages (at least 6 monthly) |  |  |
| Machinery & Equipment - Caught in moving parts - Build-up of Lint - Incorrect cycle or materials - Incorrect use of dryer |  |  |  | - Fractures - Bruises - Dislocations - Permanent injuries - Burns - Death |  |  |  | - Check that the door interlock is working i.e. door cannot be opened when in use or dryer stops operating when door is opened. - Lint filters are cleaned before each use. - Dryer is on preventative maintenance plan to ensure vents are cleaned at least annually. - Conduct inspections of hoses and vents to check for blockages (at least 6 monthly). - Ensure area around the dryer is clean and free of clutter. - Only place suitable materials in the dryer. - Never put items contaminated with flammables in the dryer e.g. solvents, oils, fats. Items must be pre-washed with detergent in hot water. - Dryer is not overloaded. - Cool down cycle is adequate to reduce the temperature of the items. Once cycle is finished, clothes should be removed and separated (not piled up). - Check weight of items being placed into dryer to ensure dryer is not overloaded. - NEVER run the dryer without anybody checking on it. - Fire extinguisher is readily available. |  |  |
| Noise - Exposure to loud noise |  |  |  | - Permanent hearing damage |  |  |  | - Install noise reducing material in the room. - Personal protective equipment e.g. ear plugs, earmuffs. |  |  |
| Extreme Temperatures - Exposure to heat |  |  |  | - Heat exhaustion - Dehydration |  |  |  | - Workers have access to cool drinking water. - Area is air-conditioned where possible. - Fans are available to encourage air movement. - Workers to have regular breaks in extreme temperatures. |  |  |
| Other |  |  |  |  |  |  |  |  |  |  |
| Review hazard / risk assessment if task or circumstances change & at intervals appropriate to the level of risk (minimum 5 years) |  |  |  |  |  |  |  |  |  |  |
| Step 4: Monitor & Review: |  |  |  |  |  |  |  |  |  |  |
| Were the controls effective? |  |  |  | Were there any unforeseen hazards / incidents? |  |  |  | New controls |  |  |
| Yes |  | No |  | Yes |  | No |  |  |  |  |