Template only MUST modify to site conditions

| Site / Area: |  |  |  | Date of Assessment |  |  |  | Risk Assessment # | 104RA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Completed by (name) |  |  |  | Signature |  |  |  |  |  |
| In Consultation with: |  |  |  | Signature |  |  |  |  |  |
| Identify / describe activity, equipment, area or event you are assessing: |  |  |  |  |  | Ride on Mower |  |  |  |
| Authorised by: |  |  |  | Signature: |  |  |  | Date: |  |
| In conjunction with this risk assessment, training / education and development of a relevant SOP may be required. |  |  |  |  |  |  |  |  |  |
| #### Step 1: Identify the hazard/s / Impact: #### What do you believe are the hazards? What could happen? |  |  |  | Step 2: Assess the potential risks: What do you believe are the risks? How could this happen? |  |  |  | Step 3: Reducing the risk: What do you believe can be done to reduce the risk? Controls |  |
| Hazardous Manual Task Sustained or awkward postures |  |  |  | - Sprains & strains |  |  |  | - Work Instructions - Two persons to conduct any field repairs |  |
| Machinery & Equipment - Faulty system - Ejected material - Refuelling while engine is hot - Refuelling while equipment is turned on |  |  |  | - Breakdown of equipment - Striking - Eye Injury - Lacerations - Fire / explosion |  |  |  | - Work Instruction - Pre-start checks including guarding, fuel and seat shut off switch - Personal Protective Equipment to be worn at all times - Safety devices, guards, switches and shields are fitted, secured and functional - Securing of work tools and equipment - Stopping mowing if another person / animal is in the vicinity - Equipment on maintenance schedule - Reporting of faults - Lock out tag out of equipment - Equipment refuelled when engine cold and turned off |  |
| Gravity - Poor housekeeping - Equipment makes unexpected movement |  |  |  | - Slip, Trip, Fall |  |  |  | - Work areas on the ground to be kept clear of trip hazards - Driving at slow enough speed to keep control over unexpected hazards - Wearing seatbelt (where fitted) while operating equipment - Travel up / down slopes rather than across and use 15 degrees as the maximum to attempt to mow. |  |
| Extreme Temperature - Hot Weather Wet Weather |  |  |  | - Sunburn - Dehydration - Cold |  |  |  | - Personal Protective Equipment to be worn at all times - Sunscreen and drinking water to be readily available at all times |  |
| Noise - Not wearing Personal Protective Equipment Equipment not maintained |  |  |  | - Hearing loss |  |  |  | - Personal Protective Equipment to be worn at all times - Equipment on maintenance schedule |  |
| Biological Insects / Wildlife |  |  |  | - Stings & bites |  |  |  | - Personal Protective Equipment to be worn at all times - First aid kit available at all times |  |
| Other |  |  |  |  |  |  |  |  |  |
| Review hazard / risk assessment if task or circumstances change & at intervals appropriate to the level of risk (minimum 5 years) |  |  |  |  |  |  |  |  |  |
| Step 4: Monitor & Review: |  |  |  |  |  |  |  |  |  |
| Were the controls effective? |  |  |  | Were there any unforeseen hazards / incidents? |  |  |  | New controls |  |
| Yes |  | No |  | Yes |  | No |  |  |  |