STANDARD OPERATING PROCEDURE

| 3D Printer |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 020SOP |  | 021RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Electricity, Extreme Temperatures, Mechanical, Fire, Hazardous Chemicals |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Check for obvious faults and defects. Check the item has a current electrical test tag attached. Faulty equipment must not be used. Immediately report it. - Ensure the risk assessment has been completed. - Ensure no slip / trip hazards are present in the workspace and walkways. - Locate and ensure you are familiar with the operation of the ON/OFF switch. - Ensure print board is securely clipped onto the platform. - Ensure the room is well ventilated with circulating air. - Printer is located on a level, stable surface. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Switch on computer and login using the credentials located on the computer. - Switch on the printer at the rear. - Open UP! 3D printer software located on the desktop. - Press the initialise button on the printers touch screen. - Ensure the printer and filament spool is positioned correctly. - Preheat the platform using the printer software. - Ensure you have sufficient material on the spool and the allocated time available for the print job. - Close doors and covers on the printer prior to starting print job. - Keep fingers and foreign objects clear of the moving printer head. - Before handling, allow sufficient time for the parts to cool once printing is complete. - Do not touch the nozzle, print board or printed model by hand. - Wear heat resistant gloves and safety glasses when handling the printer and model. - Do not remove the model from the print board while the print board is still attached to the printer. - Do not open printer doors whilst printing. - When changing a filament spool, follow the manufacturer’s extraction and retraction guidelines. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Gently guide the printing platform out once printing has finished and printer is switched off. - Save your work and shutdown the computer. - Return all tooling and safety equipment to correct storage location. - Leave the machine and work area in a safe, clean and tidy state. - Dispose of waste material appropriately. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |