STANDARD OPERATING PROCEDURE

| Air Compressor |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 041SOP |  | 038RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Unrestrained air hose whipping around; Material blown under pressure; Noise; slips, trips & falls; Hazardous Manual Tasks |  |  |  |  |  |  |  |  |
|  |  |  | Rings and Jewellry must not be worn. |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Ensure the work area is well ventilated and not interfering with other students or activities. - Visually check all air hoses and electrical leads used with the compressor for damage and slip/trip hazards. - Ensure all guards for the compressor (e.g. belt and pulley cover) are in place and in good working order. - Ensure you are familiar with the operation of the ON/OFF switch. - Check that the regulator is set to the appropriate pressure for the activity. - Check the oil level of the compressor in the oil sight glass |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Ensure all attachments used with the air compressor are in good condition before using. - Be careful when attaching and disconnecting tools to the air hose. Hold the air hose and tool firmly during this process. - Check the compressor regularly, noting pressure increase and cut-out/cut-in pressure. - Listen for any air leaks from any flexible airlines and immediately report any leaks. - Adjust pressure regulator to suit work requirements - Turn off and disconnect air hoses and electrical leads after use. - Never leave air compressor and equipment unattended. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Leave the work area in a safe, clean and tidy state – remove any waste. - Release condensation from the drain before storing away - Air hoses and electrical leads should be coiled with large loops. FORBIDDEN - Never use compressed air to clean off clothes, workbenches or floor surfaces - Never direct compressed air at your body or another person. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |