STANDARD OPERATING PROCEDURE

| Bandsaw |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 042SOP |  | 039RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Foreign body in eyes; severe lacerations / cuts; Electric shock; crush injuries. |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Rings and Jewellry must not be worn. |  |  |  |
| # PRE-OPERATIONAL SAFETY - Ensure no slip/trip hazards are present in workspaces and walkways. - Locate and ensure you are familiar with the operation of the ON/OFF starter and E-Stop. - Check that all guards are in position. - Ensure push stick is available. - Lower the blade guide and guard to full effect. - Start the dust extraction unit before using the saw. - Faulty equipment must not be used. Immediately report suspect machinery. |  |  |  |  |  |  |  |  |
| OPERATIONAL SAFETY CHECKS - Never leave the machine running unattended. - The work piece should be fed forward evenly and held firmly on the table to ensure effective control during cutting whilst keeping hands in a safe position. - Use a push stick when feeding material past the blade. - Do not force a wide blade on a cut of small radius. Use relief cuts when cutting sharp curves. - Before making adjustments switch off the saw and bring the machine to a complete standstill. - Stop the machine before attempting to back the work away from the blade. - Stop the saw immediately if the blade develops a ‘click’. Report it to your Supervisor / Manager. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Switch off the machine. - Leave the machine in a safe, clean and tidy state. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |