STANDARD OPERATING PROCEDURE

| Belt Sander - Fixed |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 045SOP |  | 041RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Abrasions; Foreign body in eyes; Electric shock; excessive dust. |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Gloves, rings and jewellry must not be worn |  |  |  |
| # PRE-OPERATIONAL SAFETY - Check workspaces and walkways to ensure no slip/trip-hazards are present. - Locate and ensure you are familiar with the operation of the ON/OFF starter and E-Stop (if fitted). - Check the table is set not more than 2mm from disc. - Check abrasive disc is in a serviceable condition. - Start the dust extraction unit before using the machine. - Faulty equipment must not be used. Immediately report suspect equipment. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY CHECKS - Allow machine to reach maximum revolutions before operating to avoid overloading. - NEVER attempt to sand very small items, lift or tilt the material while sanding. - Always place material on the table on the downward side of the disc travel and hold it securely on the table surface. - Hold material firmly against the table before applying pressure on abrasive disc. - Keep hands/fingers clear of abrasive disc while sanding. - NEVER attempt to or try to sharpen metal items. - NEVER leave the machine running unattended. - Before making adjustments switch off and bring the machine to a complete standstill. FORBIDDEN - Attempting to sand very small items - Attempting to sharpen tools - Attempting to sand metal |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Switch off the machine. - Leave the machine in a safe, clean and tidy state. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |