STANDARD OPERATING PROCEDURE

| Cold Cut Saw |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 048SOP |  | 044RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Shattering Blade; Airborne dust; Eye injuries; Contact with blade at point of operation; Material slipping in vice/clamp |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Check workspaces and walkways to ensure that no slip/trip-hazards are present. - Check that all safety guards are in position and are set to the minimum clearance from the workpiece being cut. - Locate and ensure you are familiar with the operation of the ON/OFF starter and E-Stop (if fitted). - Ensure all locks are securely tightened. - Ensure table and work area is clear of all tools, off-cut metal. - Faulty equipment must not be used. Immediately report suspect equipment. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Do not cut freehand, clamp material firmly in vice. - Allow the saw blade to obtain maximum speed before making a cut. - Make sure cooling fluid is flowing over saw blade. - Always stand to one side of the line of cut. - Never remove off cuts from the saw table while the saw is running. - Before making adjustments switch off and bring the machine to a complete standstill. - Never leave offcuts on saw table while cutting. - Do not clamp material shorter that the width of clamping vice. - Ensure long material is supported on the right side of machine. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Switch off the machine. - Leave the machine in a safe, clean and tidy state. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  | Signature: |  |  |  |  |  |