STANDARD OPERATING PROCEDURE

| Drill Press |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 053SOP |  | 050RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Entanglement in rotating spindle or drill; Eye injuries, Flying swarf or chips; Sharp edges & burrs; Hot surfaces or drill bits |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Check workspace and walkways to ensure no slip-hazards are present. - Check that the drill chuck guard is in position. - Ensure the chuck key (if used) has been removed from the drill chuck. - Locate and ensure you are familiar with the operation of the ON/OFF starter and E-Stop. - Follow correct clamping procedures to ensure work is secure. - If the job obstructs the walkway erect a barricade. - Adjust spindle speed to suit drill, cutter diameter and type of material. - Faulty equipment must not be used. Immediately report suspect equipment. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Never leave the Drill Press while it is running. - Before making adjustments or before cleaning accumulated swarf, switch off and bring the machine to a complete standstill. - Feed downwards at a sufficient rate to keep the drill cutting. - Feed with care as the drill breaks through the underside of the work. - Use a safe working posture. Beware of clothing or hair catching in the drill chuck or bit. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Never leave the Drill Press while it is running. - Before making adjustments or before cleaning accumulated swarf, switch off and bring the machine to a complete standstill. - Feed downwards at a sufficient rate to keep the drill cutting. - Feed with care as the drill breaks through the underside of the work. Use a safe working posture. Beware of clothing or hair catching in the drill chuck or bit. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |