STANDARD OPERATING PROCEDURE

| Electric Bin Lifter |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 086SOP |  | 100RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Electric shock; Repetition of movement; Awkward posture; Slip/trip/fall; Sprains/strains; Falling waste and dust; Pinch/crush in moving parts |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Read the operating manual to ensure you are familiar with the operation and parts of the bin lifter. - Before operating, the following checks should always be made. - area clear of persons and obstacles - battery voltage ok (where applicable) - brakes on castors functioning - leads and cables undamaged and untangled - ground is flat (max 6° slope) at site of operation - visual inspection of unit, ensure no parts are missing, damaged and/or unsecured. - Do not exceed the safe working load of the bin lifter. - Use caution when moving heavy (30+ kg) wheelie bins. Heavy bins can cause strain injury. - Do not use wheelie bins that do not fit the bin lifter supports or bins that are damaged or split. - Do not leave the bin lifter outdoors unnecessarily (unless it is designed for this). - Always ensure the path of travel is clear of people and obstacles when moving and operating the bin lifter. - Wear covered footwear. Secure loose clothing or jewellery that could become entangled in the lifter. Wear safety glasses to protect eyes from falling or wind-blown debris. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - STAY ALERT: Always watch what you are doing and the environment you are working in. - Only operate the bin lifter in accordance with the operating manual. - Always activate brake on castors when bin lifter is stationary. - Ensure the immediate area around and above the bin lifter is clear of persons or objects during operation. - Keep lid of wheelie bins closed during operation. - Use caution during windy conditions to contain dust, liquids, and other light waste. - Keep body clear of moving parts on bin lifter during operation. Stand to the side. Keep others clear of the lift zone. - Do not force the bin lifter. It will perform better and more safely when operated correctly. - Disconnect and turn off bin lifter when not in use. Remove the key to prevent accidental operation and ensure if the facility is available, secured to prevent unauthorised access. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Clean the bin lifter frequently to prevent the build-up of waste. See manual for method. - Check oil and battery (where applicable) levels. - Always store the bin lifter in a dry, enclosed, flat-floored area (where unit requires this). - Maintenance of hydraulic or electric parts must only be performed by qualified technicians. - Always wash your hands thoroughly after handling rubbish. All incidents must be reported immediately to your Manager/WHS Coordinator. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |