STANDARD OPERATING PROCEDURE

| Gas Barbecue |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 029SOP |  | 016RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Fire, Hazardous Chemicals, Hazardous Manual Tasks, Extreme Temperatures |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Pictogram of PPE OR cautions required |  |  |  |
| # PRE-OPERATIONAL SAFETY - Ensure you are wearing the correct Personal Protective Equipment (PPE) for the task. - Barbeque only to be used outdoors in a well-ventilated area. - Check gas bottle and barbeque for any leaks, faults or damage. Report any hazards to your supervisor or Manager immediately. - Gas Cylinders to be secured from movement and date stamped within ten (10) years. - Fire Extinguisher to be available or water hose |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Complete a visual inspection of the barbeque, looking for damage and gas leakage by spraying soapy water along the gas line and gas connections. If bubbling occurs tighten connections. - Make sure that the gas is turned on at the cylinder and that the hood and / or burner lid is up. - If fitted with a gas safety gauge, depress the button to allow gas to flow. - Push the on knob in and turn anticlockwise to the high position hold the knob in until the burner ignites. - Adjust the knob to high medium or low for the desired flame / temperature. - Use tongs to move food on the barbeque. - To turn off the barbeque turn off at the cylinder first and allow all gas left in the hose to burn off. Then turn the knob to off. - Clean the barbeque when the plates are fully cooled down. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Remove all litter from the area and ensure all equipment is clean. - When cleaning the barbeque, ensure that it has cooled sufficiently so that there is no risk of burns. - Disconnect and wipe the grease collection pipe / container / tray. - Remove any spills from the floor mat using a cloth or a mop if one is available. - Return barbeque to the designated area for storage. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |