STANDARD OPERATING PROCEDURE

| Laser Cutter |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 021SOP |  | 057RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Fire, Electricity, Hazardous Chemicals, Radiation, Extreme Temperatures and Mechanical |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Check all software and machine settings carefully before commencing any cutting operation. - Check operating lenses are clean. - Check nozzle has no build-up of material on it. - Load and position the material on the table. - Ensure the access door is closed before starting the laser cutter. - Ensure air assist has commenced automatically when machine is switched on. - Some materials may produce toxic fumes. Check Safety Data Sheet (SDS) prior to use. - Ensure power cords are positioned to avoid creating a trip hazard. - Faulty equipment must not be used. Immediately report suspect machinery to Tech Studies Coordinator. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - DO NOT place any reflective metals into the laser cabinet as the beam may be reflected out and cause injury. - DO NOT LEAVE THE MACHINE UNATTENDED WHEN IN USE - A CO2 fire extinguisher is available adjacent the machine. - Check that fumes / smoke is being extracted through the filter system when machine is operating. - Only permissible material is to be cut. If unsure if a material is suitable for laser cutting – DO NOT PROCEED, refer to operating manual. - DO NOT operate the machine if any safety features have been modified, disabled or removed. - DO NOT open the lid until the fan and light have ceased operating. - DO NOT store tools or parts on the machine. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Remove all off cuts and waste, sweep/vacuum the area. Leave the machine and work area in a safe, clean & tidy state. - Return surplus material to stock and dispose of waste material in the appropriate recycling or waste bins. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |