| Panel Saw |  |  |  |  |  | SOP Number: | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 065SOP | 066RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Cutting hands / Pinch & squash injuries / Flying chips and debris / Eye injuries / Excessive noise |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Clear the area around the machine. - Locate and check the operation of the emergency stop. - Check the blade condition and type is correct for the task. - Check riving knife is correctly set and securely mounted. - Do not store tools and parts on top of the machine. - The fence must be set for all use of the machine. - The guard should be set as low as possible. Where auto adjusting, the saw blade guard should return to the table without assistance. - Faulty equipment must not be used. |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Stand to one side when turning on power and feeding material past blade. - Never leave the machine while it is still running. - Check material being machined for nails and loose knots etc. - Do not force materials past saw blade. - Wait for the machine to stop and isolate before making any adjustments. - Do not place your hands in line with the cut. - Use a push stick for material less that 150mm wide and to remove offcuts. - Do not attempt to remove off-cuts or saw dust from the table with your hands when the blade is in motion. |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Switch off and lock out the machine. - Leave the machine in a safe, clean and tidy state. |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |

Acknowledgement much of the information in this document has been sourced by the Department of Education & Children’s Services SA.