STANDARD OPERATING PROCEDURE

| Timber Planer |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 080SOP |  | 081RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Cutting hands, Pinch and squash injuries, Flying chips and debris , Eye injuries, Excessive noise, Respiratory issues |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Check guards are in position and operational. - Locate and check the operation of the emergency stop. - Minimum length of material 300mm - Set depth of cut (maximum 3 millimetres) and lock table. - Check and lock fence in position. - One piece of material to be machined at a time. - Faulty equipment must not be used. Immediately report suspect machinery. - Do not attempt to plane end grain. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Never allow hands to pass over the cutter. - Never leave the machine while it is still running. - Wait for the machine to stop and isolate before making any adjustments. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Switch off the machine. - Leave the machine in a safe, clean and tidy state. - Reset the depth of the cut to 1mm or 0mm after use. - Reset cutter block guards. - Lockout machine. - Sweep the machine down and remove all chips, shaving and wood dust. FORBIDDEN - Planing end grain. - Planing timber 300mm or less in length |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |