STANDARD OPERATING PROCEDURE

| Timber Scroll Saw |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 090SOP |  | 081RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Oscillating sharp blades, dust and fumes, eye injuries, electricity, pinch and squash. |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Ensure no slip/trip hazards are present in workspaces and walkways. - Locate and ensure you are familiar with the operation of the ON/OFF starter and lockable E-Stop. - Check that the blade guard is securely fixed in position. - Ensure the blade teeth point downward, toward the table. - Make all adjustments only when the machine is turned off and unplugged from the power source. - Choose the correct size and style blade for the material and the type of cutting planned. - Tighten the blade securely in the chucks and adjust it in-line with the blade support. - Ensure the blade tension is properly adjusted. If "knocking" is heard, readjust the tension. - Faulty equipment must not be used. Immediately report suspect machinery. |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Never leave the machine running unattended. - Start the dust extraction unit before using the saw (if fitted). - Ensure material to be sawed is of correct size/shape, can be held safely during operation and only use materials free from defects. - Ensure that the stock rests securely on the table and never start the saw with the blade in contact with the material. - Use a safe working posture and never overreach — you may slip and fall into the machine. - Keep your hands, fingers, and body parts well out of the way of the blade. Never have your hand or fingers in the line of cut. - Give the blade time to do its job. The teeth are small and you must feed your work slowly don’t force your material into the blade. If you are cutting a radius or circle don’t turn too sharply. - Never attempt to remove timber swarf by hand. Always turn off the machine and use a brush. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Switch off the equipment, engage E-stop and reset all guards to a fully closed position. - Return all tooling and fixtures to the correct storage location. - Leave the machine and work area in a safe, clean and tidy state. - Return surplus material to stock & dispose of waste material in an appropriate recycling or waste bin. |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |