STANDARD OPERATING PROCEDURE

| Vibrating Plate Compactor |  |  |  |  |  | SOP Number: |  | RA Number: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 083SOP |  | 084RA |
| DO NOT use / operate / this equipment or complete the task unless you have been trained. |  |  |  |  |  |  |  |  |
| POTENTIAL RISKS: Manual handling injuries • Noise levels • Exhaust fumes • Vibrations • Hot engine components • Flammable fuel |  |  |  |  |  |  |  |  |
| # PRE-OPERATIONAL SAFETY - Check compactors fuel level, oil level, air filter - Check area for any potential obstacles or hazards - Ensure that no flammable gasses or in the area - Use assistance to lift the compactor - Ensure the is not a confined space or a potential confined space - Ensure you are wearing your PPE - Ensure that the machine has cooled before hosing down |  |  |  |  |  |  |  |  |
| # OPERATIONAL SAFETY PRECAUTIONS - Fuel compactor (if required) and remove fuel container back to store before starting - Turn fuel on (lever down) - Close choke and place throttle at idle - Pull rope start in a smooth action, keeping knees bent and back straight - Once started, open choke and allow to warm up - Move throttle quickly to full position - Let compactor plane along by itself, using the handles to steer the machine along. - Bring the machine back to idle to stay stationary or when travelling over compacted or solid ground. - Work progressively over the area to be compacted, to prevent the machine from ‘digging in’. - Spray area with water to suppress dust and to aid with compaction. |  |  |  |  |  |  |  |  |
| # HOUSEKEEPING - Allow compactor to idle for a few seconds before pressing the stop button. - Ensure the stop button is depressed until the motor is completely stopped - Once the motor has cooled the machine can be cleaned. - Place the machine in a secured area and back to store. - Report any fault |  |  |  |  |  |  |  |  |
| Date of last review: |  |  |  | Signature: |  |  |  |  |