PREVENTATIVE MAINTENANCE SCHEDULE

| To be customised to suit specific work environment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WEEKLY SCHEDULE |  |  |  |  |  |  |  |  | DATE: |
| PLANNED ACTIVITY |  | RESPONSIBILITY | FREQUENCY | M | T | W | T | F | COMMENTS |
| Maintenance Requests | Plan and prioritise requests for maintenance work |  | Weekly |  |  |  |  |  |  |
| Vandalism Inspection | Inspect grounds and buildings for vandalism, graffiti, or damage |  | Weekly |  |  |  |  |  |  |
| Waste Removal | Place rubbish bins out for collection |  | Weekly |  |  |  |  |  |  |
| Housekeeping | Clean and tidy up all areas: workshop, grounds |  | Weekly |  |  |  |  |  |  |
| Locks | Lubricate padlocks (with graphite) subject to weather |  | Monthly |  |  |  |  |  |  |
| Paths and Roadways | Check paths to ensure tripping hazards are repaired. |  | Weekly |  |  |  |  |  |  |
|  | Clear paths of fallen branches, rubbish etc. |  | Weekly |  |  |  |  |  |  |
| Playground Equipment | Check all equipment for splinters, loose fastenings, depth of soft fall (minimum 200mm) under possible impact areas |  | Weekly |  |  |  |  |  |  |
|  | Check for any damage to surfacing (i.e. matting) |  | Weekly |  |  |  |  |  |  |
|  | Check for foreign objects (i.e. glass, needles, animal waste, insects etc.) |  | Weekly |  |  |  |  |  |  |
|  | Rake sand pits (covered) |  | Weekly |  |  |  |  |  |  |
|  | Check damage to ancillary equipment. (bins fencing, gates etc.) |  | Weekly |  |  |  |  |  |  |
|  | Rake sand pits (early learning centre / uncovered) |  | Daily |  |  |  |  |  |  |
| Lighting | Check all external lights to ensure are working (not flickering) |  | Weekly |  |  |  |  |  |  |

| MONTHLY SCHEDULE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DATE: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PLANNED ACTIVITY |  | RESPONSIBILITY | FREQUENCY | J | F | M | A | M | J | J | A | S | O | N | D |  | COMMENTS |
| Lighting | Check interior lighting for faulty lights. |  | 6 Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Clean interior and exterior lights |  | 6 Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| External Buildings | Clean all gutters |  | 6 Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Clean out sumps, gratings, pits, and downpipes |  | 6 Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Check brickwork and external timber work |  | 6 x year |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Check concrete and pavers for movement |  | 6 x year |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Cleaning / dusting of corrugated iron |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pest control tests |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Asbestos signage is clearly displayed |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Backflow prevention devices |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Storm water pits |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plumbing | Carry out visual inspection of taps, sinks, cisterns, toilet pans, urinals and drains |  | Quarterly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Service and clean traps to kitchen sinks and hand basins |  | Quarterly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Change drinking water filters (Pura-Tap / chillers) |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Operate pressure release valves on Hot Water Units |  | 6 Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| PLANNED ACTIVITY |  | RESPONSIBILITY | FREQUENCY | J | F | M | A | M | J | J | A | S | O | N | D | COMMENTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Grounds | Check condition of fencing and gates |  | Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Automatic gates service |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Service school oval (scarify, fertilize, check for divots) |  | Quarterly |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fertilise lawns |  | 2 x year |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mow lawns |  | As required |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Weed and replace flower beds |  | As required |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Check operation irrigation sprinklers, controllers, and valves |  | Seasonally |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Check and remove low hanging or dead tree branches |  | 2 x year |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Conduct visual inspection of all buildings and external grounds |  | Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Arborist assessment conducted on trees |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Electrical | Conduct push button testing on all RCDs |  | 6 Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Check all electrical outlets for damage |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Conduct 90 minute battery discharge testing on emergency/exit lights |  | 6 Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test and tag all portable electrical equipment |  | As required |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Conduct 2 yearly performance tests on all RCDs |  | As required |  |  |  |  |  |  |  |  |  |  |  |  |  |

| PLANNED ACTIVITY |  | RESPONSIBILITY | FREQUENCY | J | F | M | A | M | J | J | A | S | O | N | D | COMMENTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal Building | Check and service all latches, locks, window winders, hinges, and door closers |  | 6 Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Check operation of windows and doors |  | 6 Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Curtains and Blinds – check and clean |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Carpet cleaning/replacement |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Check Painting/paint as required |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Check tiling/repair |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bi-fold door service |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Automatic door service |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Air Conditioner Fans / Ventilation | Clean reverse cycle air-conditioner filters |  | Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Clean exhaust fans, range-hoods, kilns |  | Quarterly |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Clean ventilators |  | Quarterly |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Service evaporative air-conditioners pre-season (insert sterilising tablet) |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Service evaporative air-conditioners post season shut-down |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Service reverse cycle air-conditioners |  | 6 Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Service and performance test fume cupboards |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Service Extraction Systems |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |

| PLANNED ACTIVITY |  | RESPONSIBILITY | FREQUENCY | J | F | M | A | M | J | J | A | S | O | N | D | COMMENTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gas / Heating Appliances | Service all gas equipment |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Service heating units |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Inspect flues for blockages or damage |  | 6 Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Check/service boiling water units |  | 6 Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fire Safety | Servicing of fire extinguishers |  | 6 Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Servicing of hose reels |  | 6 Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Servicing of fire blankets |  | 6 Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Smoke alarm systems |  | 6 Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Smoke control systems |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fire sprinkler system |  | 6 Monthly |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plant and Equipment | Service lawnmower |  | Manufacturer |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Service tractor |  | Manufacturer |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Service brush cutter |  | Manufacturer |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Service hedge trimmer |  | Manufacturer |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Service trailer |  | Manufacturer |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Service grounds utility |  | Manufacturer |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Service Lifts (goods and person) |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |
| External Fixtures | Anchor points on roof |  | Annually |  |  |  |  |  |  |  |  |  |  |  |  |  |